In order to comply with directive 2006/42/EC on machinery, please fill out this check list and save it in the technical construction file at the fabricator/installer.

Checklist for compliance with MD 2006/42/EC

(Art. REF: 062.8251.-- / 062.8252.-- / 062.8253.-- / 062.8280.--

Version: 24/10/2019

Make sure the document is dated correctly and signed.

Project name:	………………………………………………………………………………………….

Address: 	………………………………………………………………………………………….

Element ID (CE-reference):	……………………………………………………………………

1/ Onsite risk assessment before placement

| Inspection | Yes | No | Action required | Date solved |
| --- | --- | --- | --- | --- |
| Is the system (profiles and accessories) modified without approval of Reynaers? |  |  |  |  |
| Surroundings of the installation are free up to 500mm? |  |  |  |  |
| Is the sliding element an emergency exit or primary exit? |  |  |  |  |
| The glass is laminated per EN ISO 12543-1/2, or toughened per EN 12510-1? |  |  |  |  |
| The weight of the sliding elements and configuration complies with the selected motor(s) according to catalogues: 089.D25X.XX and/or 089.E25X.XX or AD.089.C34X.XX |  |  |  |  |
| Protection at danger points in non public areas: - Are a lot of the users elderly, infirm, disabled persons and/or young children (e.g. day care center, retirement home, hospital,…)? |  |  | IF YES: contactless protection (ESPE) required (suppliers: cedes.com or beasensors.com) ☐ Installed ☐ Not installed yet (as requested by customer) |  |
| - Can the door be operated remotely (e.g.building automation)? |  |  | IF YES: contactless protection (ESPE) required (suppliers: cedes.com or beasensors.com) ☐ Installed ☐ Not installed yet (as requested by customer) |  |
| - Is the speed changed above 70mm/s? |  |  | IF YES: speed needs to be configured in line with the Reynaers safety rules. ☐ Done ☐ Not in line (as requested by customer) |  |
| Is the sliding door installed in a public area? |  |  | IF YES: mark transparent leaves (e.g. suitable labels or by using colored materials) ☐ Installed ☐ Not installed yet (as requested by customer) |  |
| Is the sliding door installed in a public area? |  |  | IF YES: contactless protection (ESPE) required (suppliers: cedes.com or beasensors.com) ☐ Installed ☐ Not installed yet (as requested by customer) |  |

2/ Points of attention during placement

| Inspection | Yes | No | Action required | Date solved |
| --- | --- | --- | --- | --- |
| The installation is done according to the Reynaers specifications? This is checked and approved by the certified responsible according to the “Installation checklists”? |  |  |  |  |
| The calibration of the motor is completed successfully according to the motor manual? |  |  |  |  |
| The collision detection is tested on site and working flawlessly? |  |  |  |  |

3/ After installation

| Inspection | Yes | No | Action required | Date solved |
| --- | --- | --- | --- | --- |
| CE label is completed and fixed to the sliding window according to the instructions in the catalogue? |  |  |  |  |
| User manual is handed over to the user? |  |  |  |  |
| Checklist maintenance is handed over to the user (yearly maintenance recommended)? |  |  |  |  |
| A construction file must be kept for 10 years containing: - the results of the in situ risk assessment (this form) |  |  |  |  |
| - a copy of the installation manuals |  |  |  |  |
| - technical information element (dimensions, configuration, weight, e.g. .lob file) |  |  |  |  |
| - filled in checklists installation and maintenance |  |  |  |  |

The above inspection took place  .. / .. / ….

by  …………………                                                    	Signature:	………………………………………………

Uninstalled safety measures listed with "as requested by customer" are confirmed by customer

by  …………………                                                    	Signature:	………………………………………………