| In order to enjoy warranty for Hi-Finity, this checklist needs to be filled out by the Hi-Finity certified responsible after the installation of the element on the building site. This document needs to be saved in the technical construction file during the warranty period by the fabricator/installer. Make sure the document is dated correctly and signed. In case of failing documentation, or any element “Not OK” after actions taken, the warranty by Reynaers is no longer valid. Project name: ……………………………………………………………………………………………….. Address: ………………………………………………...………………………………………………… |  |
| --- | --- |
| Element ID: ...……………………………………………… System: HFP 147 HFP 179 Version: manual motorized | Configuration (e.g. slide – fixed – corner - …) …………………………………………………………………………………… |

| Inspection |  |  | OK | Not OK | Action required | Date solved |
| --- | --- | --- | --- | --- | --- | --- |
| Visible strip composite profile - bottom - top - vertical frame | HFP 147 8 mm 4-9 mm 4-9 mm | HFP 179 10 mm 6-11 mm 6-11 mm |    |    |  |  |
| The element is installed according to the specifications - Bottom frame profile is horizontal and straight: max deflection 2 mm over vent stroke? - Bottom frame profile is well supported as documented in the catalog? |  |  |   |   |  |  |
| Meeting section - Brush piece is well positioned, firmly fixed in the workshop, as described in the catalog? - Meeting section is well aligned with the brush piece? - Meeting section is vertical and perpendicular to bottom profile? |  |  |    |    |  |  |
| Smooth connection of rail pieces: components connected in different locations? |  |  |  |  |  |  |
| The vent slides smoothly - Brushes correctly installed? - No dragging of meeting section observed? |  |  |    |    |  |  |
| Normal sound during use? (below 70dB) |  |  |  |  |  |  |
| The lock + LED work properly and repeatedly : open and close the element several times to verify. |  |  |  |  |  |  |
| The rail is free of dust and debris? |  |  |  |  |  |  |
| The correct power supply is installed according to the prescriptions? |  |  |  |  |  |  |
| MOTORIZED VERSION |  |  |  |  |  |  |
| Checklist for compliance with MD 2006/42/EC is completed and signed? |  |  |  |  |  |  |
| The residual-current device (RCD) & mains circuit breaker (MCD) work properly? |  |  |  |  |  |  |
| ELEMENT APPROVED (required corrections implemented) |  |  |  |  |  |  |

The above inspection took place  .. / .. / ….

by  ……………………	Signature	……………………………………..